| ● ○ ● ● ○ ○ Standard Symbol Definition and Purpose | A stabilized construction access is defined by a point of entrance/exit to a construction site that is stabilized to reduce the tracking of mud and dirt onto public roads by construction vehicles. |
| --- | --- |
| Appropriate Applications | - Use at construction sites: - Where dirt or mud can be tracked onto public roads. - Adjacent to water bodies. - Where poor soils are encountered. - Where dust is a problem during dry weather conditions. - This BMP may be implemented on a project-by-project basis in addition to other BMPs when determined necessary and feasible by the Resident Engineer (RE). |
| Limitations | - Site conditions will dictate design and need. |
| Standards and Specifications | - Limit the points of entrance/exit to the construction site. - Limit speed of vehicles to control dust. - Properly grade each construction entrance/exit to prevent runoff from leaving the construction site. - Route runoff from stabilized entrances/exits through a sediment-trapping device before discharge. - Design stabilized entrance/exit to support the heaviest vehicles and equipment that will use it. - Select construction access stabilization (aggregate, asphaltic concrete, concrete) based on longevity, required performance, and site conditions. The use of asphalt concrete (AC) grindings for stabilized construction access/roadway is not allowed. - Use of constructed/manufactured steel plates with ribs for entrance/exit access is allowed with written approval from the RE. - If aggregate is selected, place crushed aggregate over geotextile fabric to at least 300 mm (12 in) depth, or place aggregate to a depth recommended by the RE. Crushed aggregate greater than 75 mm (3 inches) and smaller than 150 mm (6 inches) shall be used. - Designate combination or single purpose entrances and exits to the construction site. - Implement BMP SC-7, “Street Sweeping and Vacuuming” as needed and as required. - Require all employees, subcontractors, and suppliers to utilize the stabilized construction access. - All exit locations intended to be used continuously and for a period of time shall have stabilized construction entrance/exit BMPs (TC-1 “Stabilized Construction Entrance/Exit” or TC-3 “Entrance/Outlet Tire Wash”). |
| Maintenance and Inspection | - Inspect routinely for damage and assess effectiveness of the BMP. Remove aggregate, separate and dispose of sediment if construction entrance/exit is clogged with sediment or as directed by the RE. - Keep all temporary roadway ditches clear. - Inspect for damage and repair as needed. |

Stabilized Construction Entrance/Exit (Type 1)

Stabilized Contraction Entrance/Exit (Type 1)

Stabilized Construction Entrance/Exit (Type 2)