- Compliance Options Diagrams

- The Paper and Other Web Coating (POWC) MACT contains more than one option for compliance, each resulting in different monitoring, recordkeeping, and reporting requirements.  The Compliance Options Diagrams were developed as a road map for affected facilities or regulatory agencies to use to help them understand the applicable requirements associated with each option.

- Figure 1 – Compliance Options under Subpart JJJJ

- Figure 2 – Demonstrating Compliance When Using “As-Purchased” Compliant Coatings (Option 2 or 3)

- Figure 3 – Demonstrating Compliance When Using “As-Applied” Compliant Coatings (Option 2)

- Figure 4 – Demonstrating Compliance When Using “As-Applied” Compliant Coatings (Option 3)

- Figure 5 – Demonstrating Compliance When Tracking Total Monthly HAP Applied

- Figure 6 – Demonstrating Compliance by Using a Control Device (Option 1)

- Figure 7 – Demonstrating Compliance by Using a Combination of Compliant Coatings and Control Devices

- Figure 8 – Demonstrate Compliance by Using a Capture and Control System to Meet the Outlet Concentration Limit (Option 4)

- Figure 9 – Requirements for Solvent Recovery Devices

- Figure 10 – Liquid-Liquid Material Balance for Solvent Recovery Devices

- Figure 11 – Continuous Emission Monitoring for Solvent Recovery Devices

- Figure 12 – Requirements for Oxidizers

- Figure 13 – Requirements for Multiple Control Devices and Never-Controlled or Intermittently-Controlled Work Stations

- Figure 14 – List of Equations

- Credits:  This document was made possible through the efforts of the POWC Implementation Tool Development Partnership effort, an effort to bring together the regulated and regulatory community.  It was through a group effort that this document was developed.  The logo of the partner who was the lead for this tool is listed first below.  To see a description of our partners or to get more information about the partnership effort, see http://wwww.epa.gov/ttn/atw/powc/powcpg.html

- Figure 1

Compliance Options under Subpart JJJJ

[Back to Top]

The following compliance options can be used to limit organic HAP emissions from web-coating lines under Subpart JJJJ [§63.3320(b]

Limit emissions to no more than 5% of organic HAP applied for the month [§63.3320(b)(1)]

Limit organic HAP emissions to no more than 4% of coating applied for the month [§63.3320(b)(2)]

Limit organic HAP emissions to no more than 20% of solids applied for the month [§63.3320(b)(3)]

For Oxidizer control, limit outlet concentration to less than 20 ppmv per compound on a dry basis [§63.3320(b)(4)]

Option 1

Option 2

Option 3

Option 4

Limit emissions to no more than 2% of organic HAP applied for each month [§63.3320(b)(1)]

Limit organic HAP emissions to no more than 1.6% of coating applied for each month [§63.3320(b)(2)]

Limit organic HAP emissions to no more than 8% of solids applied for each month [§63.3320(b)(3)]

New

Sources

Existing

Sources

New

Sources

New

Sources

Existing

Sources

Existing

Sources

Demonstrate compliance by using any of these options: [§63.3370]

Use a capture system and control device, see Figure 6 [§63.3370(a)(4)(i)]

Use multiple capture and/or control devices, see Figure 6 [§63.3370(a)(5)(i)]

Demonstrate compliance by using any of these options: [§63.3370]

Use “as-purchased” compliant coatings, see Figure 2 [§63.3370(a)(1)(i)]

Use “as-applied” compliant coatings, see Figure 3 [§63.3370(a)(2)(i), (a)(2)(iii)]

Track the total monthly HAP applied, see Figure 5 [§63.3370(a)(3)]

Use a capture system and control device, see Figure 6 [§63.3370(a)(4)(ii)]

Use multiple capture and/or control devices, see Figure 6 [§63.3370(a)(5)(ii)]

Use a combination of compliant coatings and control devices, see Figure 7 [§63.3370(a)(6)(ii)]

Demonstrate compliance by using any of these options: [§63.3370]

Use “as-purchased” compliant coatings, see Figure 2 [§63.3370(a)(1)]

Use “as-applied” compliant coatings, see Figure 4 [§63.3370(a)(2)(ii), (a)(2)(iv)]

Track the total monthly HAP applied, see Figure 5 [§63.3370(a)(3)]

Use a capture system and control device, See Figure 6 [§63.3370(a)(4)(iii)]

Use multiple capture and/or control devices, see Figure 6 [§63.3370(a)(5)(iii)]

Use a combination of compliant coatings and control devices, see Figure 7 [§63.3370(a)(6)(i)]

- Figure 2

Demonstrate Compliance by Using “As-Purchased” Compliant Coatings (Option 2 or 3)

[Back to top] [See Figure 1]

If “as-purchased” compliant materials are used to demonstrate compliance with the emission limits in §63.3320(b)(2) or (3), you may use any one of the following sub-options to comply: [§63.3370(a)(1)]

Each coating material applied during the month doesn’t exceed:

0.04 kg organic HAP/kg coating material (4% wt) “as-purchased” for existing sources

0.016 kg organic HAP/kg coating material (1.6% wt) “as-purchased” for new sources

Each coating material applied during the month doesn’t exceed:

0.20 kg organic HAP/kg solids coating material (20 weight percent solids) “as-purchased” for existing sources

0.08 kg organic HAP/kg solids coating material (8.0 weight percent solids) “as-purchased” for new sources

Determine the organic HAP weight-fraction of each coating material “as-purchased” [Chi] by following §63.3360(c)(1)-(3)

§63.3360(c)(1)-(3) requires the o/o to determine the organic HAP weight-fraction “as purchased” [Chi] by doing any of the following:

Test the coating material using Method 3111

Use Method 24 to determine the volatile organic content (weight-fraction of non aqueous volatile matter) of the coating material and use this value as a surrogate for the organic HAP content2

Use formulation data to determine the organic HAP weight-fraction of the coating material3

[See Figure 3] [See Figure 4]

Determine the organic HAP weight-fraction of each coating material “as-purchased” [Chi] by following §63.3360(c)(1)-(3) [See above]

Determine the coating solids weight fraction [Csi] of each coating material “as-purchased” by following §63.3360(d)(1)-(2) [See box to right]

Determine the “as-purchased” kg HAP/kg solids coating material [Hsi] by using Equation 3 in §63.3370(b)(2)(ii)

§63.3360(d)(1)-(2) requires the o/o to determine the solids content “as purchased” [Csi] by doing any of the following:

Use Method 24 to determine the solids weight fraction of the coating material2

Use formulation data to determine the solids weight fraction of the coating material4

[See Figure 4] [See Figure 5]

or

1Follow §63.3360(c)(1)(i)-(iii) to determine the organic HAP mass fraction.  The manufacturer may perform Method 311 determination.

2Follow §63.3360(d)(1) to determine volatile organic and solids content.  The manufacturer may perform Method 24 determination.

3Formulation data must represent all organic HAP present at levels ≥ 0.1% for carcinogens and ≥1.0% for non-carcinogens in any raw material used, weighted by the mass fraction of each raw material used.  The manufacturer may provide formulation data.  In the event there is any inconsistency between Method 311 and formulation data, the results of Method 311 will govern.

4In the event there is any inconsistency between Method 24 and formulation data, the results of Method 24 will govern.

- Figure 3

Demonstrate Compliance by Using “As-Applied” Compliant Coatings (Option 2)

[Back to top] [See Figure 1]

If “as-applied” compliant coating materials is used to demonstrate compliance with the emission limits in §63.3320(b)(2), you may use any one of the following sub-options to comply: [§63.3370(a)(2)]

Each coating material as-applied doesn’t exceed: [§63.3370(a)(2)(i)]

0.04 kg organic HAP/kg coating material (4% wt) “as-applied” for existing sources

0.016 kg organic HAP/kg coating material (1.6% wt) “as-applied” for new sources

Calculate organic HAP content of materials “as-applied”, accounting for materials that are added to the coating prior to application [§63.3370(c)(1)]

Monthly average of all coating materials used doesn’t exceed: [§63.3370(a)(2)(iii)]

• 0.04 kg organic HAP/kg coating material (4% wt) for existing sources

• 0.016 kg organic HAP/kg coating material (1.6% wt) for new sources

or

2 options

Determine the organic HAP weight fraction of each coating material “as-purchased” [Chi] by following §63.3360(c)(1)-(3)

Determine the monthly average organic HAP content of all materials applied [HL] by using Equation 4 in §63.3370(c)(3)

Determine the “as purchased” organic HAP content or volatile organic content of each coating material applied by following §63.3360(c) [see box to the right] [§63.3370(c)(1)(i)]

AND

Determine any one of the following: [§63.3370(c)(1)(ii)]

Calculate the “as-applied” organic HAP content of each coating material [Cahi] by using Equation 1a1 in §63.3370(b)(1)(ii)

Calculate the “as-applied” volatile organic content of each  coating material [Cavi] by using Equation 1b2 in §63.3370(b)(1)(ii)

[See Figure 7] [See Figure 10] [See Figure 11] [See Figure 12] [See Figure 13]

§63.3360(c) requires you to do the following:

Determine the organic HAP weight-fraction of each coating material “as-purchased” [Chi] by following §63.3360(c)(1)-(3)

1If solvent or other materials are not added to the as-purchased coating material, then the as-applied organic HAP mass fraction is equal to the as-purchased organic HAP mass fraction.

2Follow this method if you choose to use the volatile organic mass fraction as a surrogate for the organic HAP mass fraction of coatings.

Figure 4

Demonstrate Compliance by Using “As-Applied” Compliant Coatings (Option 3)

[Back to top] [See Figure 1]

If “as-applied” compliant coating materials is used to demonstrate compliance with the emission limits in §63.3320(b)(3), you may use any one of the following sub-options to comply: [§63.3370(a)(2)]

Each coating material as-applied doesn’t exceed: [§63.3370(a)(2)(ii)]

• 0.2 kg organic HAP/kg coating solids “as-applied” for existing sources

• 0.08 kg organic HAP/kg coating solids “as-applied” for new sources

Monthly average of all coating materials used doesn’t exceed: [§63.3370(a)(2)(iv)]

• 0.2 kg organic HAP/kg coating solid for existing sources

• 0.08 kg organic HAP/kg coating solids for new sources

or

2 options

Determine the organic HAP mass fraction of each coating material “as-purchased” by following §63.3360(c)(1)-(3)

Determine the coating solids content [Csi] of each “as-purchased coating material by following §63.3360(d)(1)-(2)

Determine the monthly average organic HAP/kg coating solids of all materials applied [HS] by using Equation 5 in §63.3370(c)(4)

Perform all of the following:

Determine the organic HAP weight-fraction of each coating material “as-purchased” [Chi] by following §63.3360(c)(1)-(3)

Determine the organic HAP weight-fraction of each coating material “as-applied” [Cahi] by following §63.3370(c)(1)(ii)

Determine the solids content of each “as-purchased” coating material by following §63.3360(d)(1)-(2) [§63.3370(c)(2)(i)]

Calculate the solids content of “as-applied” materials [Casi]by using Equation 2 in §63.3370(c)(2)(i)

Calculate the “as-applied” organic HAP to solids ratio [Hasi] by using Equation 3 in §63.3370(c)(2)(ii) 1

[See Figure 7] [See Figure 10] [See Figure 11] [See Figure 12] [See Figure 13]

1If solvent or other materials are not added to the “as-purchased” coating materials, then the “as-applied” kg organic HAP/kg coating solids is equal to the “as-purchased” kg organic HAP/kg coating solids.

- Figure 5

Demonstrate Compliance by Tracking the Total Monthly HAP Applied

[Back to top] [See Figure 1]

If monthly allowable organic HAP applied is used to demonstrate compliance with the emission limits in §63.3320(b)(2) and (3), you may use the following option to comply: [§63.3370(a)(3)]

Demonstrate that the total monthly organic HAP applied is less than the calculated equivalent allowable organic HAP

Demonstrate that the total monthly organic HAP applied (Hm) is less than the calculated allowable organic HAP (Ha) by doing all of the following: [§63.3370(d)]

Determine the total monthly organic HAP applied [Hm] by using Equation 6 in §63.3370(d)

Determine the values needed to calculate the monthly allowable organic HAP limit [Ha] by following §63.3370(l) [See box to the right] and then

Calculate Ha based on the following (use Equation 13a for existing sources and Equation 13b for new sources):

Materials applied containing ≥ 20 mass percent coating solids complying with the kg HAP/kg solids coating limit, and

Materials applied containing < 20 mass percent coating solids complying with the kg HAP/kg coating limit

[See Figure 7] [See Figure 10] [See Figure 11] [See Figure 12] [See Figure 13]

§63.3370(l) requires the o/o to do all of the following:

Determine the mass of each coating applied on an as-purchased basis

Determine the “as-purchased” coating solids content of each coating material applied in accordance with §63.3360(d)(1)-(2)

Determine the as-purchased mass fraction of each coating material applied at greater than or equal to 20 mass percent coating solids content.

Determine the total mass of each solvent, diluent, thinner, or reducer added to the coating materials which were applied at less than 20 mass percent coating solids content.

- Figure 6

Demonstrate Compliance by Using a Control Device (Option 1)

[Back to top] [See Figure 1]

If a control device is used to demonstrate compliance with the emission limits in §63.3320(b)(1), then: [§63.3370(a)(4)(i)]

Operate a capture system and control device that meets a monthly overall organic HAP control efficiency of at least [§63.3370(e)]

95% for existing sources

98% for new sources

Determine the overall organic HAP control efficiency by doing one of the following: [§63.3370(e)]

For solvent recovery devices, follow the procedures in §63.3370(i), See Figure 9

For Oxidizer systems, follow the procedures in §663.3370(k), See Figure 12

If the affected source:

Only operates always-controlled work stations with more than one capture system or more than one control device, then demonstrate compliance using one of the following: [§63.3370(e)(1)]

Follow the procedures in §63.3370(n), See Figure 12

OR

Follow the procedures in §63.3370(p) [See box to the right]

Operates one or more never-controlled work stations or one or more intermittently-controlled work stations, then follow the procedures in §63.3370(n), see Figure 13 [§63.3370(e)(2)]

§63.3370(p) requires one of the following to demonstrate compliance on a monthly basis for each web coating line or group of coating lines controlled by a common control device:

Demonstrate the volatile organic matter collection and recovery efficiency [Rv] is at least 95% at existing sources or 98% at new sources.  Calculate Rv by following §63.3370(i)(1) (i), (iii), (v), and (vi) [§63.3370(p)(1)]

Measure the mass of each coating applied; determine the volatile organic content [Cavi] of each coating material applied; monitor the volatile organic matter recovered according to §63.3350(d); and calculate volatile organic matter collection and recovery efficiency [Rv].  See Equation 7

OR

Demonstrate the overall organic HAP control efficiency for each coating line or group of coating lines served by a common capture system is at least 95% at existing sources or 98% at new sources.  Demonstrate compliance by doing one of the following: [§63.3370(p)(2) or (3)]

Follow the procedures in §63.3370(i)(2)(i)-(iv) –

Continuously monitor the control device inlet and outlet gas stream such that the control device efficiency can by calculated using Equations 1 and 2 of §63.3360; continuously monitor operating parameters established to ensure capture efficiency; determine the percent capture efficiency according to §63.3360(f); and calculate the overall organic HAP control efficiency [R] using Equation 11

Follow procedures in §63.3370(k)(1)(i)-(iii) and (k)(2)(i) –

Determine the oxidizer destruction efficiency [E]; determine the capture system efficiency [CE]; monitor the capture and control efficiency; and calculate the overall organic HAP control efficiency [R] using Equation 11

- Figure 7

Demonstrate Compliance by Using A Combination of Compliant Coatings and Control Devices (Option 2 Or 3)

[Back to top] [See Figure 1]

If a combination of compliant coatings and control devices is used to demonstrate compliance with §63.3320(b)(2) or (3), you may use any one of the following options: [§63.3370(a)(6)]

Demonstrate that the average equivalent emission rate doesn’t exceed the allowable calculated limit [§63.3370(a)(6)(iii), §63.3370(h)]

Calculate the monthly allowable HAP emissions limit [Ha] by following §63.3370(l) See Figure 5

Then determine one of the following:

HAP emission rate based on amount of coating material applied (kg HAP/kg coating).  See Figure 3

HAP emission rate based on coating solids applied (kg HAP/kg solids).  See Figure 4

HAP emissions less than calculated allowable.  See Figure 5

Follow procedures in §63.3370(i) if a solvent recovery device is used, see Figure 9

Follow procedure in §63.3370(k) if an oxidizer is used, see Figure 12

Unless your affected source:

Operates more than one capture system, more than one control device, one or more never-controlled work stations, or one or more intermittently-controlled work stations, then follow the procedures in §63.3370(n), see Figure 13

Demonstrate that the average equivalent emission rate doesn’t exceed the following on a monthly basis: [§63.3370(a)(6)(ii), §63.3370(g)]

0.04 kg organic HAP/kg coating material “as-applied” for existing sources

0.016 kg organic HAP/kg coating material “as-applied” for new sources

Demonstrate that the average equivalent emission rate doesn’t exceed the following on a monthly basis: [§63.3370(a)(6)(i), §63.3370(f)]

0.2 kg organic HAP/kg solids material “as-applied” for existing sources

0.08 kg organic HAP/kg solids material “as-applied” for new sources

Method 1

Method 2

Method 3

Figure 8

Demonstrate Compliance by Using a Capture and Control System to Meet the Outlet Concentration Limit (Option 4)

[Back to Top] [See Figure 1]

If an oxidizer is used to demonstrate compliance with §63.3320(b)(4) and you choose to meet the outlet organic HAP concentration limit, you must use the following option: [§63.3370(a)(4)(i)]

Demonstrate that the organic HAP concentration at the outlet of the oxidizer is no greater than 20 ppmv by compound an a dry basis [§63.3370(a)(4)(i)]

§63.3370(e)(3) requires a facility to install a Permanent Total Enclosure (PTE) around the coating line and to do the following:

Demonstrate that a PTE is installed and meets the requirements in §63.3360(f)(1)

AND

Determine the organic HAP concentration at the outlet of the PTE using one of the following:

Determine the control device efficiency using Equations 1 and 2 of §63.3360 and the applicable test methods and procedures specified in §63.3360(e)

Use a CEMS to determine the organic HAP emission rate according to paragraphs §63.3370(i)(2)(i) through (i)(2)(x)

- Figure 9

Requirements for Solvent Recovery Devices

[Back to top] [See Figure 6] [See Figure 7]

If a solvent recovery device is used as a control device to demonstrate compliance with§63.3320(b)(1), (2) or (3), you may use one of the following options to comply: [§63.3370(i)]

Perform a liquid-liquid material balance on a monthly basis [§63.3370(i)(1)]

Continuously monitor emissions of the capture system and control device performance [§63.3370(i)(2)]

See Figure 10

See Figure 11

- Figure 10

Liquid-Liquid Material Balance for Solvent Recovery Devices

[Back to top] [See Figure 9][Back to Figure 13]

If liquid-liquid material balance is used to demonstrate compliance of your solvent recovery control device, you must do all of the following: [§63.3370(i)(1)]

Measure the mass of each coating material applied on the coating line or group of coating lines that are controlled during the month [§63.3370(i)(1)(i)]

Determine the volatile organic content [Cavi] of each coating material “as-applied” during the month by following §63.3360(d) [See Fig 3] [§63.3370(i)(1)(iii)]

Measure and monitor the amount of volatile organic matter recovered for the month by following §63.3350(d)(2) [see box to the right] [§63.3370(i)(1)(v)]

Calculate the volatile organic matter collection and recover efficiency using Equation 7

Calculate the organic HAP emitted during the month using Equation 8

If you’re demonstrating compliance with any of the following, then determine the organic HAP content of each coating material “as-applied’ [Cahi] during the month by following §63.3360(c) [See Figure 3], then calculate the organic HAP emission rate based on coating material applied using Equation 10: [§63.3370(i)(1)(ii)]

The organic HAP emission rate based on solids applied

The organic HAP emission rate based on material applied

Emissions less than the calculated allowable organic HAP

If you’re demonstrating compliance with any of the following, then determine the solids content of each coating material applied during the month by following §63.3360(d) [See Figure 4], then calculate the organic HAP emission rate based on coating solids applied using Equation 9:  [§63.3370(i)(1)(iv)]

The organic HAP emission rate based on solids applied

Emissions less than the calculated allowable organic HAP

If you’re demonstrating compliance by limiting emissions to less than the allowable, determine the monthly allowable organic HAP emissions following §63.3370(l).  [See Figure 5]

[See Figure 7] [Back to Figure 13]

§63.3350(d)(2) requires you to do all of the following:

Install, calibrate, maintain, and operate according to the manufacturer’s specifications a device that indicates the cumulative amount of volatile mater recovered by the solvent recovery device on a monthly basis

Certify (through the manufacturer) that the device is accurate to within ±2.0% by mass

- Figure 11

Continuous Emission Monitoring for Solvent Recovery Devices

[Back to top] [See Figure 9]

If a solvent recovery device is used as your control device and you demonstrate compliance with the use of a Continuous Emission Monitoring (CEM) device, you must do the following: [§63.3370(i)(2)]

Demonstrate initial compliance by conducting a performance test on capture efficiency [§63.3360]

Install Continuous Emission Monitors (CEMs)

Continuously monitor capture system operating parameters by following §63.3370(i)(2)(i)-(vii)

§63.3370(i)(2) requires all of the following: [§63.3370(i)(2)(i)-(iv)]

Continuously monitor the gas stream entering and exiting the control device to determine the total organic volatile matter mass flow rate [§63.3370(i)(2)(i)]

Calculate the percent control efficiency [E] of the control device each month by using Equation 2 in §63.3360(e)(1)(ix) [§63.3370(i)(2)(i)]

Determine the percent capture efficiency [CE] by complying with §63.3360(f) [§63.3370(i)(2)(iii)]

Calculate the overall organic HAP control efficiency [R] each month using Equation 11 in §63.3370(i)(2)(iv)

Calculate the organic HAP emitted during the month using Equation 12, in §63.3370(i)(2)(viii)

AND, do the following, if applicable

If you’re demonstrating compliance with any of the following, then determine the organic HAP content of each coating material “as-applied’ [Cahi] during the month by following §63.3360(c) [See Figure 3], then calculate the organic HAP emission rate based on coating material applied using Equation 10: [§63.3370(i)(2)(vi)]

The organic HAP emission rate based on solids applied

The organic HAP emission rate based on material applied

Emissions less than the calculated allowable organic HAP

If you’re demonstrating compliance with any of the following, then determine the solids content of each coating material applied during the month by following §63.3360(d) [See Figure 4], then calculate the organic HAP emission rate based on coating solids applied using Equation 9:  [§63.3370(i)(2)(vii)]

The organic HAP emission rate based on solids applied

Emissions less than the calculated allowable organic HAP

If you’re demonstrating compliance by limiting emissions to less than the allowable, determine the monthly allowable organic HAP emissions following §63.3370(l).  [See Figure 5]

[See Figure 7] [See Figure 13]

- Figure 12

Requirements for Oxidizers

[Back to top] [See Figure 6] [See Figure 7]

If an oxidizer is used as your control device, you must do the following [§63.3370(k)(1)]

§63.3370(k)(1) requires all of the following:

Determine the oxidizer destruction efficiency [E] by complying with §63.3360(e) [§63.3370(k)(1)(i)]

Determine the percent capture efficiency [CE] by complying with §63.3370(f) [§63.3370(k)(1)(ii)]

Continuously monitor the operating parameter established in §63.3350(e) and (f) to ensure capture efficiency [§63.3370(k)(1)(iii)]

Calculate the overall organic HAP control efficiency [R] each month using Equation 11 in §63.3370(i)(2)(iv)

AND, do the following, if applicable

If you’re demonstrating compliance with any of the following, then determine the organic HAP content of each coating material “as-applied’ [Cahi] during the month by following §63.3360(c) [See Figure 3]: [§63.3370(k)(1)(v)]

The organic HAP emission rate based on solids applied

The organic HAP emission rate based on material applied

Emissions less than the calculated allowable organic HAP

If you’re demonstrating compliance with any of the following, then determine the solids content of each coating material “as-applied” during the month by following §63.3360(d) [See Figure 4]:  [§63.3370(k)(1)(vi)]

The organic HAP emission rate based on solids applied

Emissions less than the calculated allowable organic HAP

If you’re demonstrating compliance by limiting emissions to less than the allowable, determine the monthly allowable organic HAP emissions following §63.3370(l).  [See Figure 5]

[See Figure 7] [See Figure 13]

Demonstrate initial compliance by conducting a performance test on capture and control device efficiency [§63.3360(e) and (f)]

Continuously monitor capture and control device operating parameters by following §63.3370(k)(1)(i)-(vi) [see box to the right]

- Figure 13

Requirements for Multiple Control Devices and Never-Controlled or Intermittently-Controlled Work Stations

[Back to top] [See Figure 6] [See Figure 7]

If more than one capture system, more than one control device, one or more never-controlled work stations, and/or one or more intermittently-controlled work stations are used, than one of the following options can be chosen to comply: [§63.3370(n)]

Use a solvent recovery system and perform liquid-liquid material balance [§63.3370(n)(1)]

See Figure 10

Use a solvent recovery system and using performance testing with CEMs [§63.3370(n)(2)]

See Figure 11

Use an Oxidizer [§63.3370(n)(3)]

See Figure 12

Use uncontrolled coating lines [§63.3370(n)(4)]

Determine the organic HAP applied using Equation 6.  This is equal to the organic HAP emitted

Then determine one of the following:

HAP emission rate based on amount of coating material applied (kg HAP/kg coating).  See Figure 3

HAP emission rate based on coating solids applied (kg HAP/kg solids).  See Figure 4

HAP emissions less than calculated allowable.  See Figure 5

Determine the mass of all coating materials “as-applied” in intermittently –controlled work stations operating in by-pass mode and the mass of all coating “as-applied” on never-controlled work stations during the month.

Determine the mass of all coatings “as-applied” on intermittently-controlled work stations operating in a controlled mode and the mass of all coatings “as-applied” on always-controlled work stations during the month

Calculate the organic HAP emitted during the month using Equation 14

Then determine one of the following:

HAP emission rate based on amount of coating material applied (kg HAP/kg coating)

HAP emission rate based on coating solids applied (kg HAP/kg solids)

HAP emissions less than calculated allowable

Calculate the organic HAP emitted during the month using Equation 15

Then determine one of the following:

HAP emission rate based on amount of coating material applied (kg HAP/kg coating).  See Figure 3

HAP emission rate based on coating solids applied (kg HAP/kg solids).  See Figure 4

HAP emissions less than calculated allowable.  See Figure 5

Solvent recovery using liquid-liquid material balance

Solvent recovery using CEM’s or Oxidizer

- Figure 14

List of Equations

[Back to top]

Equation 1a – Cahi = ChiMi+j=1qChijMijMi+j=1qMij				[See Figure 3]

Equation 1b – Cavi = CviMi+j=1qCvijMijMi+j=1qMij					[See Figure 3]

Equation 2 – Casi = CsiMi+j=1qCsijMijMi+j=1qMij					[See Figure 4]

Equation 3 – Hsi = CahiCasi					[See Figure 2] [See Figure 4]

Equation 4 – HL = i=1pChiMi+j=1qChijMij−Mvreti=1pMi+j=1qMij			[See Figure 3]

Equation 5 – HS = i=1pChiMi+j=1qChijMij−Mvreti=1pCsiMi+j=1qCsijMij			[See Figure 4]

Equation 6 – Hm = i=1pChiMi+j=1qChijMij−Mvret			[See Figure 5] [See Figure 13]

Equation 7 – RV = Mvr+Mvreti=1pCviMi+j=1qCvijMij×100				[See Figure 10]

- Figure 14 (continued)

List of Equations

Equation 8 – He = 1−Rv100i=1pChiMi+j=1qChijMij−Mvret		[See Figure 10]

Equation 9 – L = Hei=1pCsiMi+j=1qCsijMij				[See Figure 10]

Equation 10 – S = Hei=1pMi+j=1qMij					[See Figure 10]

Equation 11 – R = ECE100					[See Figure 6] [See Figure 11]

Equation 12 – He = 1−Ri=1pCahiMi−Mvret				[See Figure 11]

Equation 13a – Ha = 0.20i=1pMiGiCsi+0.04i=1pMi1−Gi+j=1qMLj		[See Figure 5]

Equation 13b – Ha = 0.08i=1pMiGiCsi+0.016i=1pMi1−Gi+j=1qMLj		[See Figure 5]

Equation 14 – He = i=1pMCiCahi1−Rv100+i=1pMBiCahi−Mvret		[See Figure 13]

Equation 15 – He = i=1pMCiCahi1−R100+i=1pMBiCahi−Mvret		[See Figure 13]